General Product Questions

- What is CharCoat CC Cable Coating?

  - It’s a water-based, intumescent fireproof coating that stops fire spread on electrical cables and maintains their functionality for up to 120 minutes.

- How does CharCoat CC work?

  - It expands (intumesces) in fire conditions, forming an insulating layer that prevents flames from spreading and protects cables from extreme heat.

- What makes CharCoat CC different from other fire-resistant coatings?

  - It’s non-derating, UV-stable, waterproof, and has a low dry film thickness (1.6mm) compared to conventional coatings.

- Does CharCoat CC completely stop fires?

  - It prevents flame propagation on cables, buys critical evacuation time, and reduces secondary fire risks but does not eliminate fire hazards completely.

- What are the key benefits of CharCoat CC?

  - Prevents flame spread, maintains cable ampacity, is weatherproof, UV-resistant, and complies with international fire safety standards.

- Is CharCoat CC only for fire safety, or does it have other benefits?

  - Besides fire protection, it enhances cable lifespan, protects against UV degradation, and reduces maintenance costs.

- Does it require special handling or protective gear during application?

  - No, it’s a water-based, non-toxic product and requires only standard PPE like gloves and safety glasses.

- Can CharCoat CC be removed or reapplied?

  - Yes, it can be removed with high-pressure water jets or mechanical scrubbing, and reapplication is easy.

- Does CharCoat CC come in different colors?

  - It is available in white, but can be tinted up to 5% or top-coated with acrylic water-based paint for identification.

- Is it tested and certified?

- Yes, it meets IEC 60331-11/21, ASTM E84, BS 476, FM 3971-1992 and multiple fire safety and environmental standards.

Application & Installation

- How is CharCoat CC applied?

- It can be sprayed, brushed, or rolled onto cables. Airless spray is preferred for larger areas.

- What equipment is needed for spraying?

- Airless spray units (minimum 200 bar pressure) with tip sizes 0.017” – 0.019” are recommended.

- How thick should the coating be?

- 1.6mm dry film thickness (DFT), applied in two coats (3.0mm wet application each).

- How long does it take to dry?

- Touch dry in 2 hours, full cure in 24-48 hours (depends on humidity and temperature).

- Does it require a primer?

- No, CharCoat CC is a standalone coating and does not require primers.

- Can it be applied over dirty or oily cables?

- No, cables must be clean, dry, and free of oil or dust for optimal adhesion.

- Does CharCoat CC affect the flexibility of cables?

- No, it is highly flexible and does not crack or peel under normal cable movement.

- Can it be applied on live cables?

- It is recommended to apply it on de-energized cables, but it can be applied on live cables with proper safety measures.

- Can CharCoat CC be applied to metal cable trays?

- Yes, it should be applied to cables and the inner surfaces of cable trays for complete fire protection.

- Does it require a topcoat for outdoor use?

- No, CharCoat CC is 100% UV-resistant and weatherproof, so no topcoat is needed.

Performance & Fire Safety

- What is the fire rating of CharCoat CC?

- It is rated up to 120 minutes at 750°C - 1100°C under IEC 60331-11/21.

- Can it prevent cables from catching fire?

- It prevents flame spread but does not make cables fireproof.

- How does it compare to fire-resistant cables?

- It provides a cost-effective alternative by fireproofing standard cables instead of replacing them with expensive fire-resistant cables.

- Can it be used in hazardous areas?

- Yes, it is used in oil & gas, power plants, chemical industries, and high-risk environments.

- Does it stop toxic smoke emissions?

- Yes, it minimizes smoke generation and toxic fume emissions, making evacuations safer.

- Can it be used in underground tunnels or metros?

- Yes, it is ideal for transport infrastructure where fire safety is critical.

- What happens to cables coated with CharCoat CC in a fire?

- The coating swells (intumesces) to form a protective carbon char layer, reducing heat transfer to the cable.

- Is it resistant to short circuits and arc flashes?

- It does not prevent electrical faults but reduces fire risk due to short circuits.

- Can it withstand high-voltage environments?

- Yes, it has been tested on HV power cables up to 8000V.

- How does it perform against molten metal drips?

- It prevents molten substances from spreading flames in case of a fire.

Durability & Environmental Protection

- Is CharCoat CC waterproof?

- Yes, it is impermeable and water-resistant.

- Is it resistant to chemicals and oils?

- Yes, it resists oils, fuels, and various industrial chemicals.

- How long does the coating last?

- It extends cable life by up to 25 years with proper maintenance.

- Is CharCoat CC rodent-resistant?

- Yes, it is tested and proven to be rodent-resistant.

- Does it contain hazardous materials?

- No, it is asbestos-free, halogen-free, mercury-free, and non-toxic.

Commercial & Supply Chain Questions

- What industries use CharCoat CC?

- Oil & gas, power plants, commercial buildings, tunnels, metros, and data centers.

- Is CharCoat CC available globally?

- Yes, it is available in multiple countries, including India through Krishaa Passive Fire Protection.

- What are the packaging options?

- 22.5 kg (5-gallon pails), with custom sizes on request.

- How should CharCoat CC be stored?

- Keep it between 5°C and 32°C, away from direct sunlight and frost.

- What is the shelf life?

- 18 months from the date of manufacture.

- Is it easy to transport?

- Yes, it is a non-hazardous material, making transport and handling simple.

- What is the typical lead time for bulk orders?

- Standard orders are fulfilled quickly, with bulk orders depending on project requirements.

- Does CharCoat CC require certification before use in India?

- It meets international fire safety standards, and local approvals can be obtained as needed.

- Do you offer technical support?

- Yes, Krishaa provides full technical guidance, demonstrations, and project consultation.

- Can you conduct fire tests or demos for our project?

- Yes, fire tests can be arranged based on project needs.

- Where can I buy CharCoat CC in India?

- Krishaa Passive Fire Protection is the official licensed distributor.

- Can I become a distributor?

- Yes, we are open to new distribution partnerships.

- What are the pricing and MOQ?

- Pricing depends on volume and project scope. Contact our sales team for details.

- Do you offer site visits and audits?

- Yes, our team can assess your site and recommend customized solutions.

- How do I place an order?

- Connect with our sales team at Krishaa for order placement and technical support.